Star Visual Card Reader/Writer

Sample program with OCX driver ver1.90

TCPOcxDemo.exe Operation Manual

- Before install the TCPOcxDemo.exe, please install the VisualCardOCX1.5.0.

After installation of the OCX Driver, select “REGISTER” to finish the installation.

- After installation of the sample program by setup.exe, you can execute below application.

- How to Use (Each Button)

A (communication condition setting) --- You need to select the communication setting from this menu first of all.

Com Port No. Setting (ZUN Model)

Baud Rate Setting (ZUN Model)

Please select baud rate as dummy data even use USB I/F, any baud rate is OK.

IP Address Setting (ZL Model)

B (communication setting) ---	Communication conditions are shown at here.

C (OCX Ver.) 		---	OCX version is shown at here.

D (arbitrarily command)	---	You can input arbitrary command. For example, in case of RESET command sending, you just input “5F” to this text box.

E (Binary)		---	You can send binary data to check this box.

ex) 01H  02H,  Please type $01$02 in parameter box.

You can enter binary data and decimal data mixture in parameter box.

F (Data parameter box)	---	You can input arbitrary parameter if the command requires the parameter. For example, in case of [printing data setting], input “41” to “C“, and input like “2,0,23,PRINT DATA STRING” to this text box.

G (time out) 		---

Time length to be time out for above arbitrarily command sending is defined by this box. It becomes a parameter of Communication Method of OCX.

H (DSR check) 		---

When you check DSR check at here, then application check the DSR status of TCP RS232C communication whether TCP is busy status or not. It becomes a parameter of Communication Method of OCX.

I (executing button)	--- When you click this button, command and parameter are sent to TCP.

J (cancel communication) --- When you click, communication between PC to TCP is canceled.

K (Magnetic stripe data reading)

When you click this button, command 22H(Magnetic stripe data reading) will send from application without timeout parameter. Read data is shown on “S”.

L (Reset)

When you click this button, command 5FH (Reset) will send from application with 30 seconds timeout parameter.

M (Buffer clear)

When you click this button, command 40H (Buffer clear) will send from application with 30 second timeout parameter.

N (Send Text Data)

When you click this button, command 41H (Send Text Data) will send from application with 5 second timeout parameter.

O (Print)

When you click this button, command 46H (execution of printing) will send from application with 30 second timeout parameter.

P (Card Eject)

When you click this button, command 50H (Card Eject) will send from application with 5 second timeout parameter. At the same time, the time from command sending to receiving the response from TCP is measured. And taken time will be shown on “M”.

Q (command)

The command which was sent to TCP is shown at here.

R (response code)

It shows a response code (status) from TCP.

S (data from TCP)

It shows a response data from TCP. For example, magnetic stripe data.

T (response code)

It shows a response code (status) from OCX.